Test 3 Questions Insolation: How is it measured

- What are the 3 assumptions for STC?

Irradiance of 1000 W/m2 , Cell temperature at 25C and Air Mass at 1.5

- Describe the angle of incidence.

The angle that light strikes the surface of the PV panel.

- Describe the cosine effect?

The output of the PV panel is related to the angle of incidence following the cosine of the angle.

- Describe the Air Mass ratio.

Relationship between the distance the sunlight travels in relation to the distance at the equator. What is reflected radiation?

- Describe the Zenith angle?

Zenith is referred to the position of the sun during the daily position referenced to (the vertical) directly overhead.

- Describe the Azimuth angle?

Azimuth is referred to the position of the sun referenced to the direction at which the PV panel is pointing.

- In the northern hemisphere the sun is where?

South.

- In the southern hemisphere the sun is where?

North.

- Depending where you are on earth the sun path can move up and down the?

Horizon.

- Draw The Air Mass ratio.

- Draw the Zenith angle.

- Draw the Azimuth angle.